Urban Heat Island Mapping

City of Tampa, FL  ·  QGIS Portfolio Project 01

| Tools | QGIS 3.34, GDAL/OGR, PyQGIS, Python 3.11 |
| --- | --- |
| Data Sources | Landsat 8 (USGS EarthExplorer), US Census TIGER/Line 2022, NLCD 2021, OpenStreetMap |
| CRS | WGS84 (EPSG:4326) → Florida State Plane East (EPSG:2236) |
| Status | Complete |
| Repo | github.com/[your-username]/qgis-urban-heat-island |

## Overview

This project maps land surface temperature (LST) variation across Tampa's urban core using Landsat 8 multispectral imagery and ancillary vector data. The goal was to identify which census tracts experience the most significant urban heat island (UHI) effect — where urban materials like concrete and asphalt absorb and re-emit heat far more than surrounding vegetated areas — and to correlate that effect with land cover composition.

The output is a set of cartographic products intended for a city planning audience: a tract-level choropleth of LST deviation, a paired NDVI map showing green canopy density, and a composite risk scoring table combining both variables with impervious surface cover percentage.

This was a solo end-to-end project: raw raster acquisition, preprocessing, analysis, and final map production were all completed in QGIS, with supporting data cleaning in Python using geopandas and pandas.

## Background and Motivation

Urban heat islands are well-documented in dense metros, but the intra-city distribution of heat stress is uneven and often correlates with historical disinvestment patterns. Tampa was chosen because its mix of high-density urban core, mid-century residential suburbs, and significant impervious cover (parking lots, highway corridors) makes it a textbook UHI candidate.

The Landsat 8 thermal infrared sensor (Band 10) provides LST data at 100m resolution (resampled to 30m). While not fine-grained enough for block-level analysis, it works well for tract-level aggregation and trend mapping.

## Data Sources

| Dataset | Source | Format | Notes |
| --- | --- | --- | --- |
| Landsat 8 OLI/TIRS Collection 2 | USGS EarthExplorer | GeoTIFF | Scene LC08_L2SP_017040_20230714 (July 2023, <5% cloud cover) |
| Census Tract Boundaries | US Census TIGER/Line 2022 | Shapefile | Hillsborough County, FL |
| Land Cover (NLCD 2021) | USGS Multi-Resolution Land Characteristics | GeoTIFF | Used for impervious surface % extraction |
| Road Network | OpenStreetMap via QuickOSM plugin | GeoPackage | Tampa metro bounding box |
| Administrative Boundaries | Florida Geographic Data Library | Shapefile | City limits, county boundary |

NOTE  Scene selection criteria: acquired June–August to capture peak heat conditions; cloud cover < 5%; no atmospheric anomalies in QA band.

## Methodology

### 1. Data Acquisition and CRS Setup

Downloaded the Landsat 8 Level-2 scene from EarthExplorer (Product ID: LC08_L2SP_017040_20230714_20230721_02_T1). Level-2 products include surface reflectance and surface temperature corrections already applied by USGS.

All layers were reprojected to Florida State Plane East (EPSG:2236) — a metric projection appropriate for this region — before any spatial operations. Working in a projected CRS matters here because zonal statistics and area calculations must be done in a flat coordinate system, not geographic degrees.

gdalwarp -s_srs EPSG:4326 -t_srs EPSG:2236 -r bilinear \
  LC08_L2SP_017040_20230714_ST_B10.TIF \
  outputs/LST_B10_FL_StatePlane.tif

Census tract and NLCD layers were reprojected using QGIS Processing > Reproject Layer. Confirmed alignment by overlaying all layers and checking that tract boundaries snapped cleanly to the raster grid.

### 2. LST Derivation from Landsat Band 10

Landsat Collection 2 Level-2 ST products ship with a scale factor applied. Raw DN values must be converted to Kelvin, then Celsius:

LST (Kelvin) = DN × 0.00341802 + 149.0
LST (Celsius) = LST (Kelvin) − 273.15

Applied using the QGIS Raster Calculator. Verified output range (should be approximately 20°C–55°C for a summer Tampa scene) before proceeding.

### 3. NDVI Calculation

NDVI (Normalized Difference Vegetation Index) uses near-infrared (Band 5) and red (Band 4) reflectance:

NDVI = (Band5 − Band4) / (Band5 + Band4)

Same scale factor conversion applied to reflectance bands first. NDVI output ranges from −1 to +1; values above 0.3 generally indicate healthy vegetation cover in this climate.

### 4. Zonal Statistics — Joining Raster Values to Census Tracts

Used QGIS Zonal Statistics (Raster > Zonal Statistics) to aggregate mean LST and NDVI per census tract. Output fields appended to the tract attribute table:

- lst_mean — mean LST in Celsius per tract

- lst_stddev — standard deviation of LST within tract

- ndvi_mean — mean NDVI per tract

- imperv_pct — impervious surface % from NLCD (separate zonal stats run)

### 5. LST Deviation Calculation

Rather than mapping raw LST (which varies by season), each tract's deviation from the county mean was calculated — how much hotter or cooler relative to the Hillsborough County baseline:

lst_deviation = lst_mean − county_mean_lst

Computed in QGIS Field Calculator. County mean LST calculated as the mean of all tract means.

### 6. Risk Score Composite

Combined three normalized variables into a single 0–100 composite risk score per tract:

| Variable | Weight | Rationale |
| --- | --- | --- |
| LST deviation | 50% | Primary heat stress indicator |
| Impervious surface % | 30% | Structural driver of UHI effect |
| NDVI (inverted) | 20% | Low vegetation = reduced cooling capacity |

Each variable was min-max normalized to 0–1 before weighting. Risk tiers: Low (0–25), Moderate (25–50), High (50–75), Very High (75–100).

### 7. Symbolization and Map Production

Choropleth symbolization used a 5-class Jenks Natural Breaks classification on lst_deviation. Color ramp: diverging blue-to-red (ColorBrewer RdBu reversed), centered at 0°C deviation.

Print layout built in QGIS Layout Manager at A3 (420×297mm) landscape, 300 DPI. Layout elements: main map frame, inset overview map, legend, scale bar, north arrow, data source block, and a bar chart showing the top/bottom 10 tracts by LST deviation.

## Key Findings

- Highest LST deviations (+6°C to +9°C above county mean) clustered around downtown/Channelside, the area north of I-4 near Ybor City, and commercial/industrial zones along US-301.

- Lowest deviations (−3°C to −5°C) corresponded with tracts containing Hillsborough River State Park, parts of USF campus, and residential parcels with mature tree canopy.

- NDVI and LST showed a moderately strong negative correlation (r = −0.61 across tracts), consistent with published literature.

- Impervious surface % was the strongest individual predictor of elevated LST in this dataset.

## Limitations

- July 2023 scene captures a single point in time. Multi-date compositing would produce more stable LST estimates.

- Zonal statistics use unweighted mean per tract; tracts with large non-residential land (parks, industrial zones) may have skewed means.

- Landsat 8 at 100m/30m resolution cannot resolve fine-scale heat patterns within a single census tract.

- NLCD 2021 impervious layer is two years older than the imagery; recent development not captured.

## Troubleshooting Log

### Issue 1 — NULL values in zonal statistics after reprojection

After reprojecting the NLCD impervious raster from Albers Equal Area (EPSG:5070) to Florida State Plane (EPSG:2236), zonal statistics returned NULL for ~12% of tracts.

Cause: Reprojection introduced NoData edge artifacts where the raster extent didn't fully cover coastal tract polygons.

Fix: Buffered the NLCD raster extent by 500m before reprojecting, then clipped to county boundary after. NULLs resolved.

### Issue 2 — LST Raster Calculator returned implausible values

Raster Calculator expression for LST conversion returned values in the range 2000–5000 (clearly wrong).

Cause: Scale factor had already been applied in the Level-2 product; I was applying it a second time.

Fix: Reviewed Landsat Collection 2 product guide. Level-2 ST_B10 delivers scaled integers; the scale factor (0.00341802) and additive offset (149.0) are applied once to get Kelvin. Corrected expression.

### Issue 3 — GEOID field lost after zonal statistics join

Census tract attribute table lost the GEOID field after the zonal statistics join, breaking the subsequent join to demographic data.

Cause: QGIS zonal statistics creates a copy of the input layer; the field was renamed with a prefix.

Fix: Renamed fields in Field Calculator. For future runs: rename fields immediately after zonal stats before any additional joins.

## Repository Structure

qgis-urban-heat-island/
├── README.md
├── data/
│   ├── raw/
│   │   ├── landsat/          # Landsat 8 GeoTIFF bands (B4, B5, B10, QA)
│   │   ├── census/           # TIGER/Line tract shapefiles
│   │   └── nlcd/             # NLCD 2021 impervious surface raster
│   └── processed/
│       ├── LST_B10_FL_StatePlane.tif
│       ├── LST_celsius.tif
│       ├── NDVI.tif
│       ├── tracts_zonal_stats.gpkg
│       └── tracts_risk_scores.gpkg
├── scripts/
│   ├── lst_conversion.py
│   ├── ndvi_calculation.py
│   ├── zonal_stats_batch.py
│   └── risk_score_normalize.py
├── qgis-project/
│   └── tampa_uhi.qgz
├── outputs/
│   ├── maps/
│   │   ├── tampa_uhi_choropleth_A3.pdf
│   │   ├── tampa_uhi_choropleth_A3.png
│   │   └── tampa_ndvi_paired_map.png
│   └── data/
│       └── tract_risk_scores.csv
├── docs/
│   ├── workflow_notes.md
│   ├── crs_decisions.md
│   └── symbolization_notes.md
└── .gitignore

## Skills Demonstrated

- Raster analysis: band math, raster calculator expressions, resampling, reprojection

- Zonal statistics: aggregating raster values to vector polygon attributes

- CRS management: WGS84 to projected CRS, troubleshooting reproject artifacts

- Field calculator: normalization formulas, composite scoring

- Cartographic design: diverging color ramps, Jenks classification, A3 print layout

- PyQGIS scripting: batch processing, field manipulation

- Workflow documentation: real-time notes, troubleshooting log

License: Data sources are publicly available under USGS, US Census, and OSM open licenses. Code: MIT. Map outputs: CC BY 4.0.